Test Plan & Results

### 1. Test Plan

| # Test | # Method | # Expected Result |
| --- | --- | --- |
| # Motion Trigger (Hardware/Logic) | # Walk past the robot at a normal walking pace from approximately 2 meters away. | # The PIR sensor detects the movement, triggering the ESP32 to immediately open the eyelids and begin the left-right scanning sequence. |
| # Audio Interrupt (Hardware/Logic) | # Clap hands or speak loudly while the robot is actively scanning the room. | # The sound sensor picks up the spike, temporarily pausing the eye sweep to turn the head servo and pulse the NeoPixels bright green. |
| # Idle Timeout (Logic) | # Leave the room entirely and time the robot’s behavior with a stopwatch. | # After exactly 60 seconds of no PIR activity, the NeoPixels shut off and the servos quietly reset to their centered/closed resting positions. |
| # Intuitive Interaction (Usability) | # Invite a user into the room without explaining how the robot works, simply asking them to "explore the piece." | # The user should naturally realize that their physical presence wakes the robot, and they should eventually discover that it responds to sound. |

### 2. User Testing

Who were the users? We tested the installation with Vibhor, Aakash, and Manan. They are all 1st-year undergraduate students at BITS Design School. Testing with peers who have a design background but no prior knowledge of our specific code or hardware allowed us to see how naturally the interactive elements communicated themselves.

What tasks were given? Because this is a reactive installation rather than a traditional product, we didn't give them a strict step-by-step manual. We gave them exploratory prompts:

- Walk up to the installation and observe what happens.

- Try to hold the robot’s attention.

- Figure out if the robot can do anything else besides looking left and right.

### 3. Results

| # Test | # Outcome | # Pass/Fail |
| --- | --- | --- |
| # Motion Trigger | # The PIR sensor was highly reliable. It caught all three users as soon as they crossed the threshold, waking the robot up in under a second. | # Pass |
| # Audio Interrupt | # Triggered perfectly when Vibhor clapped his hands loudly. However, it failed to register when Manan simply snapped his fingers or spoke in a normal voice. | # Pass (Functional) / Fail (Sensitivity) |
| # Idle Timeout | # The timer logic worked flawlessly. After the users left, the system returned to its dormant state right at the 1-minute mark. | # Pass |
| # Intuitive Interaction | # Waving and moving around was intuitive for everyone. But 2 out of the 3 users did not realize they could use sound to interact until we hinted at it. | # Mixed / Needs Improvement |

### 4. Observations

User behavior: The initial reaction was great. All three users were visibly surprised when the scrap pile "woke up" and opened its eyes—it definitely hit the design intent of creating unexpected life. Once the eyes started tracking, they naturally tried moving side to side to see if the robot was following them. Interestingly, Aakash tried to physically touch the head to see if it would react, rather than making a noise.

Pain points: The biggest pain point is that the sound interaction is somewhat hidden. Because the servo motors make a slight buzzing noise when they move, the users assumed the robot was just a mechanical visual piece. They didn't think to talk to it or make sharp noises. Additionally, the physical potentiometer on our sound sensor was dialed way too low, meaning it only picked up very aggressive noises rather than casual interaction.

### 6. Improvements Suggested

Based on testing:

- Hardware Adjustment: We need to take a small screwdriver and turn up the sensitivity dial directly on the LM393 sound sensor module. A regular finger snap or a spoken "Hello" needs to be enough to trigger the head turn, not just a loud clap.

- Visual Cues: We might adjust the code so the NeoPixels "breathe" very slightly when the robot is awake. This pulsing light might invite the user to try different forms of interaction (like sound) rather than just standing still.