Software Documentation

### Software Overview

What does the software do? The software acts as the behavioral engine for the robot, constantly reading environmental inputs to trigger specific physical reactions. Its main job is to manage a polling loop that watches for human presence.

When someone walks by, the code wakes the robot up, sequences the eyelid and eyeball servos to perform a scanning motion, and simultaneously listens for loud noises. If a noise occurs during this active window, the software interrupts the standard routine to move the head servo and fire off a custom LED light pattern.

Finally, it keeps track of idle time, changing the robot's status lights to red if the room stays empty for over a minute.

Platforms and Tools Used:

- Language: MicroPython

- Libraries: Built-in machine (for PWM and GPIO control), time (for non-blocking delays and timers), and neopixel (for LED string manipulation).

- Development Environment: MicroPython IDE (Thonny) used to flash the script onto the ESP32 hardware.

### 2. System Logic

Here is the step-by-step breakdown of how data moves through the system during operation:

- The system powers on, initializing all pins and moving servos to their default closed/centered positions.

- The ESP32 continuously checks the digital state of the PIR sensor.

- A person walks in front of the robot, causing the PIR sensor to send a HIGH signal.

- The ESP32 reads this signal, resets its internal idle timer, and turns the NeoPixels dim green.

- The microcontroller sends PWM signals to the eyelid servos, commanding them to open.

- The microcontroller commands the eyeball servos to look left.

- While looking left, the ESP32 rapidly polls the sound sensor for one second.

- If a sharp sound is detected, the ESP32 immediately updates the Neo Pixels to pulse bright green and commands the head servo to turn left.

- The eyes then scan right, and the system repeats the sound-check process for the right side.

- Once the scanning sequence finishes, the ESP32 commands the lids to close and resets all motors to the center.

### 3. Flowchart

Visual representation of logic